DEPT. OF AERONAUTICAL ENGINEERING

SAMPLE UNIVERSITY QUESTIONS

Class: B.E – Vth Semester					Subject: Aerodynamics-II

Subject code: AE2303

- Write the one-dimensional energy and momentum equations for an adiabatic compressible steady flow.

- What is under-expanding nozzle flow?

- What are the properties of flow medium on which the velocity of sound through the medium depends upon?

- Write internal energy equation for one dimensional high speed flow in general partial differential form.

- What are the properties of flow medium that determine the velocity of sound wave in the medium?

- Write down the Bernoulli’s equation for compressible flow.

- Explain zone of action and zone of silence for a body moving at a speed of sound.

- Explain Mach number spectrum.

- What is isentropic compressibility?

- With a suitable sketch illustrate the propagation of waves from a sound source moving at a speed of sound.

- Why is a convergent divergent nozzle required to expand a flow from stagnation condition to supersonic velocity?

- Explain the phenomenon of choking in a nozzle.

- Explain why a converging diverging configuration is required for the acceleration of flow from subsonic to supersonic conditions						(8)

- What are the flow losses that are suffered by a compressible flow in variable area ducts? How does the back pressure affect the losses?					(8)

- Derive an expression for choked mass flow rate through a converging diverging nozzle in terms of total pressure, total temperature and throat area.				(10)

- Sketch the pressure variation along the centerline of a converging diverging nozzle for optimum expansion. What is the influence of back pressure on this variation?	(6)

- Obtain an expression for velocity of sound on terms of specific heats and local temperature in air medium from one dimensional continuity, momentum and energy equations. 										(8)

- Derive relationship between the ratio of stagnation pressure to static pressure and Mach number for an isentropic flow. Derive similar relations for temperature and density ratios.																						(8)

- Derive a general expression for the speed of sound in a compressible gas from first principle.										(10)

- Derive an expression for the correction factor to be applied to pitot-static probe readings in compressible subsonic flows.							(6)

- Derive energy equation for one-dimensional steady compressible flow from first principle. Deduce the isentropic relation between stagnation and sonic conditions for temperature, pressure and density.							(10)

- Air flow is discharged to sea level atmosphere through a sonic nozzle. If the air storage pressure at the reservoir is 40x105 N/m2, determine the pressure, temperature density at the exit of the nozzle. Assume that the reservoir air is at ambient temperature.	(6)

- Derive an expression connecting area and velocity variations with Mach number for a one-dimensional compressible flow.							(8)

- Explain with suitable sketches the effect of pressure ratio on flow in a Laval nozzle.(8)

- Sketch the pressure variation along a Laval nozzle for different back pressures.	(6)

- An airplane flying at 1500 kmph at an altitude where the pressure and temperature are respectively 3x104 N/m2 and -50 degree C. Calculate the pressure, density and temperature at the leading edge of the wing.						(10)

- What is expansion hodograph?

- Give any two practical examples of interaction and reflection of shock waves.

- Under what conditions an attached shock wave to solid body like wedge is detached?

- What is shock polar?

- Bring out two important differences between Rayleigh Flow and Fanno Flow.

- What is Rayleigh correction formula for pressure measurements in supersonic flows?

- What are the applications of Rankine-Hugoniot relation?

- How is the strength of a shock wave determined in a supersonic flow?

- Bring out any two important differences between shock waves and expansion waves in a supersonic flow.

- Illustrate Prandtl-Meyer expansion round a convex corner with a neat sketch.

- What is Rayleigh’s correction for total pressure measurement in supersonic flows? Why is the correction required?							(8)

- With a neat sketch, explain Prandtl-Meyer expansion. Derive an expression for Prandtl-Meyer function.								(8)

- Bring out the essential differences between Rayleigh flow and Fanno flow. Give at least two examples for each type of flow						(8)

- With a neat illustration distinguish between reflection and refraction of shock waves.

(8)

- With a neat sketch explain the concept of Prandtle-Meyer expansion waves. How do flow properties like total pressure and Mach number change across the expansion

waves. 											 (8)

- What conditions favour detachment of shock waves in supersonic flows over solid bodies?										(8)

- What is shock polar? What is its use in supersonic aerodynamics?			(6)

- Explain pressure deflection diagrams							(4)

- Explain briefly the procedure to be followed for the design of a supersonic nozzle using method of characteristics.								(6)

- Derive a relation connecting flow turning angle, shock angle and free stream Mach number for oblique shock waves.							(8)

- If a compression corner of angle 20 degree is allowed to encounter an uniform stream of supersonic flow at Mach 5, calculate the shock wave angle and pressure & Mach number behind the shock wave.								(8)

- Explain shock reflection from a solid boundary with a suitable sketch.		(6)

- An incident shock wave with wave angle=35 degree impinges on a straight wall. If the upstream flow properties are M1 = 3, P1 =1 atm, T1 =300 K, calculate the reflected shock wave angle with respect to the wall.							(10)

- What is the importance of Rankine-Hugoniot relationship for shock waves?	(4)

- Sketch the shock polar for M=2.5 and explain the method of finding the Mach numbers and shock angles for a turning angle of 5 deg.					(8)

- Explain why shocks cannot occur in subsonic flows					(4)