# AMD Mantle benchmark 26.03.2014.

# with Catalyst 14.3beta

Testing was done on the following PC configuration:

CPU: AMD Phenom II x4 945 @ 3000 MHz
COOLER: Arctic Cooling Freezer 64pro PWM + Arctic Cooling MX-2
RAM: 5GB DDR2 800MHz (1x1GB Buffalo + 2x1GB Crucial + 1x2GB KingMax) 
MBO: AsRock A770DE+ (bios 1.70)
GPU: MSI AMD Radeon HD 7770 OC 1GB GDDR5 @ 1020/1125 MHz
Soundcard: Asus Xonar DG 5.1
PSU: Corsair CX 400W
SSD: Kingston SSDNow V300 120GB @ SATA-II
HDD: Samsung SpinPoint F3 1000GB 7200rpm 32MB S-ATA II & Seagate SV35 7200.2 160GB 7200rpm 8MB 
CASE: Chieftec Giga DF-01B
FAN: 2x Cooler Master Silent fan SI2 + 1x Spire 120mm 1500rpm @ 9,5V + 1x Xilence RedWing 120mm 1200rpm hydro
DVD: LiteOn (Phillips-BenQ) DVD-RW DH20A4P
OS: Windows 8.1 Professional x64 CRO with Media Center
Monitor: Samsung SyncMaster 2253BW 1680x1050 @ 60Hz
Keyboard: Logitech G710+
Mouse: SpeedLink Kudos
Mousepad: Cooler Master Storm Tactics DC CS-M FPS
UPS: C-Lion Blazer Vista 800VA 480W
Speakers: Logitech S-220 2.1
Headset: Creative Fatal1ty
Webcam: Logitech C210 720p HD

Validations:

CPU-Z: http://valid.canardpc.com/sf72e9

GPU-Z: http://www.techpowerup.com/gpuz/77s7s/

Speccy: http://speccy.piriform.com/results/LJ4PZMFYEG25Hts5fTjiLcC

Tests used:

- Thief (2014) internal benchmark on very low and high graphical settings with Mantle and without. Benchmark ran 3 times, best results taken.

- StarSwarm Benchmark with scenario Follow and with the low and high graphical settings, both Mantle and D3D. Each benchmark run 2 times, better result taken.

- Battlefield 4 multiplayer with map Siege of Shanghai with 64 players after the Levolution event, a 5 minute run with settings set to LOW.

- Battlefield 4 Test Range run (shooting a 30 bullet magazine into targets, taking down a boat with the stationary HMG, taking down targets and barrels with the second magazine and handgun, running into the building, using a flashbang inside the building, full circle arround the island in a atack boat with some shooting, running inside a Little Bird, flying straight up with the Little Bird and then falling to the top of the building with a parachute) both with DX11 and Mantle on LOW and HIGH settings.

- Preliminary results (frames per second):

- As it can be seen, Mantle helps when running Thief on lowest settings, but on higher settings the difference between DirectX11 and Mantle is so low that it can be discarded as an in-benchmark variation and nothing more.

- The same could be said for StarSwarm. On low settings Mantle wins, on High settings Mantle “does nothing”.

- But in Battlefield 4 the story is quite different. Mantle does not help in any situation. When analysing the frametime logs (made with the PerfOverlay.FrameFileLogEnable comamnd) it is clearly seen that there are persistant huge performance dips (fps drops), mostly while shooting into targets.

- The frame time graphs are shown in the next section.

- Battlefield 4 benchmark results:

- Shanghai, 64p, Low settings, DX11, 1680x1050

- Shanghai, 64p, Low settings, MANTLE, 1680x1050

- When compairing DirectX with Mantle on the same graph:

- Blue - DirectX			Green - Mantle

- Blue - DirectX			Red - Mantle

- Test Range, Low settings, DirectX, 1680x1050

- Test Range, Low settings, Mantle, 1680x1050

- When compairing DirectX with Mantle on the same graph:

- Green - Mantle			Red - DirectX

- Test Range, High settings, DirectX, 1680x1050

- Test Range, High settings, Mantle, 1680x1050

- When compairing DirectX with Mantle on the same graph:

- Green - Mantle				Red - DirectX

- Thief benchmark results:

- DX11 High

- Mantle High

- DirectX very low

- Mantle very low

- Star Swarm on low settings

- Star Swarm on High settings:

- Conclusion:

- Thief (2014) - Mantle does a decent job. Increases performance in bottlenecking situations.

- Star Swarm - Mantle does a decent job.

- Battlefield 4 - Mantle does the opposite of a decent job, makes the game run allmost unplayable.

- After some researching, CPU&GPU usage monitoring, Battlefield 4 performance overlay graph monitoring and evaluation of so obtained data, this can be concluded:

- There is no bottleneck in the test system when running Thief on higher settings. On lower settings there is a minor cpu bottleneck when running the game in DX mode. Mantle helps in that scenario.

- There is no detectable bottleneck in the test system when running Star Swarm. Mantle increases performance on low settings and keeps the same performance on High settings as DirectX but with more objects shown on the screen (weird).

- There is a high CPU bottleneck when running Battlefield 4 multiplayer in DirectX, mostly noticable on larger maps (like Siege of Shanghai) with many players. GPU usage goes 60-70% on Siege of Shanghai with 64 players, can’t go higher and has drops bellow 50%. The framerate is barely playable (30-50 fps mostly).

- With Mantle the GPU usage increases a lot, framerate varie mostly between 50 and 80 but there are many many fps drops to 2-3-4 fps that make the game stutter and decrease the average framerate (when calculated) to about 55. Those constant framedrops make the game unplayable when using Mantle and it seems they are caused by large CPU FrameTime spikes visible on the PerfOverlay graph.

- On higher settings, Mantle goes crazy, consumes huge quantities of VRAM and reduces the framerate to almost half of the framerate when running the game in DX11 with the same high settings.

- In Battlefield 4 Mantle IS broken and it needs repairs and optimisations, but it has potential.